Technical Specifications of  Blood Transportation Vans as approved by the Technical Committee for Specifications of ‘Equipments & Diagnostics for Blood Banks’ in its meeting held on 13th May 2008.

| 1. Engine: | Diesel- 4 Cylinder, 4 strokes, DI |
| --- | --- |
| 2. Emission norms: | Compliant with Bharat Stage- III |
| 3. Capacity: | 2500-3500 cc |
| 4. Maximum output: | 45-60 kw |
| 5. Maximum torque: | 175-216 nm@1750-2200 rpm |
| 6. Chassis frame: | Either Monocoque or Bicoque body |
| 7. Body dimension | a. Overall length: 4350-5300 mm b. Overall width: 1650-2100 mm c. Overall height: 2500-2800mm Aspect d. Ratio (Height: Width): 1.2 to 1.3 e. Ground clearance: 160-210 mm f. Floor loading height: Maximum of 850 mm g. Adequate internal space to accommodate 4 (four) blood Transportation boxes as per specification. |
| 8.Storage compartment dimension | Volume: Minimum of 8 cubic meters Length: Minimum of 2800 mm Height: Minimum of 1755m |
| 9. Electrical System: | 2 nos. 12v batteries, 5 AH |
| 10. Alternator: | minimum of 90 AMPs @ 13.5volts |
| 11. Fuel tank capacity: | Adequate for travelling 400 kms without refuelling |
| 12. Payload capacity: | Minimum of 1400 kgs |
| 13. Vehicle Exterior | a. Doors: Dual hinged doors with 270 degree open out at the rear, with glass view panels. Automatic door closures to be provided b. Windows: Two quarter plate tinted windows minimum 1500m from the ground level c. Step well: One step well (1600m long x 100mm wide) 400mm high at the rear door for easy exit and entry into the vehicle |
| 14. Seating in driver compartment: | One bucket seat for driver and a bench type seat for two assistants |
| 15. Accessories: | Spare wheel, vehicle tool kit, etc. |
| 16. Vehicle Standard items: | Should be as the ARAI standards |
| 17. Gross Vehicle weight: | Minimum 3000 kgs. |
| 18. Air Conditioning: | To enable temperature of 250C + 20C |
| 19. Power steering: | Should be provided |
| 20. Tyres: | Regular Tyre-Tube/Tubeless |
| 21. Power Mode: | a) AC b) DC with charger |
| 22. Vehicle | The vehicle should have provision of fittings for fixing Four (4) Blood Transportation Boxes with external dimension of each transportation box within 1000 -1500mm (L)X 800-1000 mm (W)X 600- 700 mm (H). After the award of the contract, the supplier should liaise with supplier of such boxes for fittings and electrical connections etc. |